NAME _____ALEXANDER J KING___________________

- In large AC machines the armature is located in the _____________?

  - Stator

  - Rotor

- In AC machines, what type of rings are used on the rotor?

  - Split Rings

  - Slip rings

- Generators convert ______________ energy into _______________ energy?

  - Electrical: Mechanical

  - Mechanical: Electrical

  - Rotational: Mechanical

  - Mechanical: Rotational

- The frequency output of an AC generator is dependent upon?

  - The frequency of the applied voltage

  - The size of the generator

  - The number of poles and rotational speed

  - Where the armature is located

- A common frequency in the aircraft industry is 400Hz, determine how fast a 6-pole generator must spin to produce this frequency.

  - 1000rpm

  - 2000 rpm

  - 4000 rpm

  - 8000 rpm

- If a generator spins at 1800 rpm and outputs a 60Hz frequency, how many poles must be present in the generator?

  - 1

  - 2

  - 3

  - 4

- A generator with 8 poles spins at 2000 rpm, what is the frequency of the output voltage?

  - 133.3Hz

  - 16.7Hz

  - 60Hz

  - 120Hz

- The process in which a small amount of power is sent to the field winding to build the magnetic field  of the generator is known as ________________ or ____________________?

  - Self-Excitation: Boot Strapping

  - Boot Strapping: Back EMF

  - Back EMF: Self-Excitation

  - Counter EMF: Back EMF

- An AC induction motor has a synchronous speed of 1800 rpm, if underload, the motor runs at 1750 rpm. Determine the percentage of slip present in the motor.

  - 2.78%

  - 5.76%

  - 9.72%

  - 10.3%

- What is a main draw back of an AC induction motor?

  - Large

  - Not self-starting

  - Produce less power

  - Expensive

- Maximum starting torque occurs when currents I1 andI2 are _________ degrees out of phase.

  - 0

  - 90

  - 180

  - 270

- What type of component is used to achieve the phase shift between I1 andI2?

  - Resistor

  - Inductor

  - Capacitor

  - Centrifugal Switch

- In a split-phase induction motor, what component ensures the auxiliary winding is not damaged after start-up

  - Disconnect

  - Breaker

  - Fuses

  - Centrifugal Switch

- At what % of nominal motor speed should the centrifugal switch open at?

  - 20%

  - 50%

  - 75%

  - 100%

- There are three main starting methods for induction motors covered in the lecture, capacitive start, split-phase, and _______________.

  - Bump Start

  - Soft Start

  - Kick Start

  - Shaded Pole